// Traffic Light Controller Module

module traffic_light_basic (

input clk,

input reset,

input farm_car,

output reg R1, Y1, G1, R2, Y2, G2

);

reg [1:0] state;

// State encoding

parameter S0 = 2'b00; // Farm Green, Highway Red

parameter S1 = 2'b01; // Farm Yellow, Highway Red

parameter S2 = 2'b10; // Highway Green, Farm Red

parameter S3 = 2'b11; // Highway Yellow, Farm Red

always @(posedge clk or posedge reset) begin

if (reset)

state <= S2; // Start with highway green

else begin

case (state)

S0: state <= S1;

S1: state <= S2;

S2: state <= S3;

S3: state <= farm_car ? S0 : S2;

default: state <= S2;

endcase

end

end

always @(*) begin

// Turn off all lights by default

R1 = 0; Y1 = 0; G1 = 0;

R2 = 0; Y2 = 0; G2 = 0;

case (state)

S0: begin R1 = 1; G2 = 1; end

S1: begin R1 = 1; Y2 = 1; end

S2: begin R2 = 1; G1 = 1; end

S3: begin R2 = 1; Y1 = 1; end

endcase

end

endmodule

// Test Bench for Traffic Light Controller

module tb_traffic_light_basic;

reg clk;

reg reset;

reg farm_car;

wire R1, Y1, G1, R2, Y2, G2;

// Instantiate the traffic light controller

traffic_light_basic uut (

.clk(clk),

.reset(reset),

.farm_car(farm_car),

.R1(R1), .Y1(Y1), .G1(G1),

.R2(R2), .Y2(Y2), .G2(G2)

);

// Clock generation: 10ns period

initial begin

clk = 0;

forever #5 clk = ~clk;

end

// Stimulus process

initial begin

// Initial reset

reset = 1;

farm_car = 0;

#10 reset = 0;

// Simulate farm car detection

#40 farm_car = 1;

#40 farm_car = 0;

// Continue simulation

#100 $finish;

end

// Print header once at start

initial begin

$display("\nTime(ns) | farm_car |  R1  Y1  G1  |  R2  Y2  G2");

$display("------------------------------------------------");

end

// Print signal states at every positive clock edge

always @(posedge clk) begin

$display("%8t |    %b    |  %b   %b   %b  |  %b   %b   %b",

$time, farm_car, R1, Y1, G1, R2, Y2, G2);

end

endmodule